CaseIH Pro 700 / Kinze Planter Quick Reference Guide

Basics of operating a Pro 700 for VR planting with a Kinze planter are listed below.  There are many other planter and monitor settings that must be completed for the system to VR plant.  Tips on Page 2.

NOTE: With monitor versions >V26 and “Auto-Reload RX” option selected, you no longer have to re-select the RX each time you cycle power to the monitor.   Previous versions did not “remember” the Rx file when you cycled power, but if you activate this option it appears to keep the chosen Rx active.  This is a great improvement over previous versions.

- When attaching the Planter to Activate the Task Controller

  - Home Screen

  - Press TOOLBOX

    - Press TC (tab at bottom)

      - Ensure Task Controller is INSTALLED

      - Ensure Auto-Resume is ENABLED

    - This is will require you to restart the monitor

- When starting up the machine/monitor after Task Controller has been installed

  - Home Screen

  - Press PERFORMANCE

  - Press TC (tab at bottom)     [note: your Task can NOT be active to change fields]

    - Set Grower, Farm, Field.

    - Crop Type: set to Corn or Soybeans

    - Task: you may need to press Resume to activate the task to make changes below

      - System should automatically create a new Task name, this is ok

  - Press TC Implement

    - Should default to show your planter.

      - If planter doesn’t show up, press inside the white box and select Kinze ____

  - Press TC MAP

    - Rate Group 1 Actual Rate should default into top line

    - TC Prescription

      - Press inside box to select the Prescription

  - Press TC PF

    - Verify implement width is correct

  - Press TC Data

    - Set Hybrid/Variety

    - Cycle through to make sure other info is correct under TC DATA

- Planting

  - Home Screen

  - Press RUN

    - RUN 3 is set up with your Virtual Terminal for the Kinze Planter

      - Press the Crop Selection Screen on the right (will probably say CORN)

        - Verify the correct crop is selected (colored in blue)

        - Verify the row spacing (30” for Corn and 15” for beans [interplant])

        - Press Prev to go back

      - Verify the Section Control button is ON

      - Press the POP button on the right to verify population settings

        - Flat Rate Planting: verify the BASIC box is selected (colored in blue)

        - VR Planting: verify the PRESCRIPTION box is selected (colored in blue)

          - This must be selected for VR Planting

      - Press the Crop Selec

- Press TASK (tab at bottom)

- Press Run.  Up to 6 Run Screen options are displayed.

  - Seed Ctrl is displayed on one of the Run screens.  This MUST show Seed Ctrl (A) for the planter to apply using the Prescription.  Tap on the Rate (35K for Corn, for example) and make sure Auto is highlighted in Black.  Press Enter to save your changes.

  - Run 4 usually contains the mapping display.  Map can be on other Run screens as well.

  - On the Map screen, tap Layer in the top right corner.

    - Data Type: Seed

    - Zoom: Multiple settings.  Field locks the view to the extents of the field.

    - Bound: Shows field boundary

    - Marks:  Will display field marks if any have been made in the current field.

    - Data Style: Color/Gray

    - Rx Style:  Color will “fill in” the rate change polygons.  Outline will show the outline of the rate changes only.

  - Tap Legend.  The user must set up the legend for the Prescription.  Tap in each color to set the range you’d like to see.  Press OK.

- General Pro 600 Monitor Settings:

  - Toolbox

    - Impl tab

      - Verify correct width

    - Product

      - Very important settings here.

        - Product Name: if using this option, enter your variety name here.

        - Form: Seed

        - Crop: Corn or Soybeans

        - Default App Rate:  enter the Default Rate you’d like to plant if you lose GPS signal or want to flat rate plant a field

        - Delta App Rate: set to 1.  If flat rate planting, rates with change by 1000 sds/acre if you adjust rates manually.

        - Min App Rate: set to 2000 seeds below your lowest Rx rate.  IF THIS IS NOT SET CORRECTLY, YOU WILL NOT VR PLANT.  This is the lowest rate the planter will apply.  It MUST be set to a value LESS THAN your lowest Rx rate.  For example – if 28K is your lowest rate, then set to 26K.

        - Max App Rate:  set to 2000 seeds above your highest Rx rate.  Same logic as above – just reversed.

        - Package Size: N/A

        - RX Scale Factor: Do not adjust.  Default is 1:1.000.  If you needed to cut your Rx rate in half, for example, you could set this to 1:0.500.